/////////////////////////////////////////////////////////////////////////

//Emma Clason

//MTE Associates Class of 2025

//B-Braun Team //March 3rd, 2025

//LAST UPDATED: 03/03/2025

//ver 1.0 /////////////////////////////////////////////////////////////////////////

//SETUP

//Import necessary Libraries

//Initialize Zed 2 Camera

//Initialize OpenCV for Image Processing

//Initialize Pixel Length Measurment Library

//Establish Constants

realStripeDistance = X //in millimeters

needleLength = 25.4 //1 inch in millimeters

realPixelDistance = 0 //real distance between catheter stripes

//Establish Variables

currentPixelDistance = 0 //current distance between catheter stripes

insertionAngle = 0 //calculated angle of catheter in degrees for ease of use

needleTipDepth = 0 //calculated needle tip depth in millimeters

//Function to find insertion angle

function calculateAngle(realPixelDistance, currentPixelDistance)

return arccos(currentPixelDistance / realPixelDistance) * (180 / PI)

//Function to calculate needle point depth function

calculateDepth(insertionAngle, projectedNeedlePoint)

return projectedNeedlePoint * sin(insertionAngle * (PI / 180))

//MAIN LOOP: while True: //Capture frame from Zed 2 camera

//Detect the two black stripes on the catheter

//Calculate the pixel distance between the stripes

//Check if the catheter is in the "zeroing" position

if inZeroingPosition():

// Set the initial pixel distance for reference

print("Zeroing complete. Initial pixel distance set to:", realPixelDistance)

//Calculate the insertion angle

insertionAngle = calculateAngle(realPixelDistance, currentPixelDistance)

//Project the needle tip position

projectedNeedlePoint = (needleLength / realStripeDistance) * currentPixelDistance

//Calculate the needle tip depth

needleTipDepth = calculateDepth(insertionAngle, projectedNeedlePoint)

//Display the found values on the screen, including insertion angle, needle tip depth

//Break the loop if the user takes catheter out of frame